QUALITY CONTROL SYSTEMS  /  EVIDENCE-BASED TECHNICAL REVIEW

# CVC Set / Complete Kit — Revised In-Process Control Record Strategy

CVC Set / Complete Kit

| Issue status | PROPOSED CONTROLLED DRAFT — NOT APPROVED |
| --- | --- |
| Date prepared | 22 Oct 2026 |
| Document type | Word process-control report / companion to six independent XLSX forms |
| Controlled source | WI-DD-02 Rev.3 |
| Assessment basis | IN-PROCESS ONLY — not final inspection/release |
| Use restriction | For QC/QA review only. No product acceptance or release permitted on draft alone. |

Evidence rule: “Required” is not “available”. No gap is verified closed without approved documentation and objective records.

# 1. Operational record structure

| ID | Record title | Original split preserved | Applicability |
| --- | --- | --- | --- |
| F-823-i13 | CVC Set | Single-sheet production/assembly checklist; 25 numbered original controls | CVC only |
| F-823-i13-2 | CVC Complete Kit | Two-page item/kit checklist; 44 numbered original controls | CVC only |
| F-823-i15 | Injection moulding & extrusion | Common line record with 21 original defect/weight legend items | Shared |
| F-823-i27 | 3-Way Stopcock Assembly | 15 original checks and manual leak-screening conditions | Shared / 3-way |
| F-823-i4-7 | Neil Pack packaging | 3 process parameter controls and 16 final checks | Shared |
| F-823-i4-1 | Zip Lock bags | Four blow machines plus four bag sealing/cutting machine groups | Shared |

# 2. Field requirements for revised operational record

Product/part and approved BOM/specification revision; lot, production work order, date, machine/shift, material lot and operator/QC.

Characteristic plus ORIGINAL COMPANY acceptance criterion (with unresolved criteria visibly indicated); approved verification route and controlled specification ID.

Actual time-stamped observation/measurement, equipment ID/calibration status, accept/reject/pending, remarks, sample identity and applicable evidence reference.

Signature/review fields remain blank for actual authorized review. A proposed draft or blank test report cannot count as executed evidence.

No change to approved sample interval, process limits or AQL before QC/QA/Engineering authorization.

# 3. Gap-to-form mapping

| Gap | Priority | Operational form IDs | Change needed / alternative evidence |
| --- | --- | --- | --- |
| CVC-G01 | P1 | i13, i13-2 | For the applicable test: Component/test piece ID; diameter (mm); standard/method ID; specification number/rev; required peak tensile force (N); observed peak force (N); equipment/calibration; specimen and lot ID; Pass/Fail/Pending; protocol/results attachment. If separately verified through qualification or periodic evidence, record that reference and do not force an |
| CVC-G02 | P2 | i13, i13-2 | Lumen ID; catheter OD (mm); manufacturer claim (ml/min); minimum (ml/min); actual (ml/min); method ID; instrument; sample count; report/lot evidence reference; result. |
| CVC-G03 | P2 | i13, i13-2, i15 | Add inspected component, instrument ID, 2.5x or greater magnification, defect finding, measured result where applicable, time/shift, disposition and lot traceability. |
| CVC-G04 | P2 | i13, i15 | In-process record: component/material revision and approved qualification record ID; no routine X-ray test unless control plan requires it. |
| CVC-G05 | P1 | i13, i13-2 | Control reference: Annex C liquid test report and Annex D aspiration test report by product family/lot/design; optional in-process screening result and parameters recorded separately. |
| CVC-G06 | P1 | i13, i13-2 | Record guidewire component code/lot, supplier/qualification report IDs, approved dimensions and evidence verification status; visual in-process result remains separate. |
| CVC-G07 | P1 | i13, i13-2 | Component code, OD (mm), source spec, Annex I evidence reference; if performed, applied force (N), observed separation and Pass/Fail. |
| CVC-G08 | P1 | i13, i13-2 | Needle nominal size/lot, bonding evidence, patency method/qualification ID, source approval, receipt disposition. |
| CVC-G09 | P1 | i13, i13-2 | Record dilator size/lot, applicable Table 2 band, tensile test/qualification ID, component conformity status and traceable report. |
| CVC-G10 | P2 | i13, i13-2 | Record syringe supplier/lot/capacity/spec revision and qualification report reference; keep in-process movement/marking outcome separate. |
| CVC-G11 | P1 | i13, i13-2, i27 | Luer type, material, component lot and supplier, approved drawing/revision, type-test reference, periodic check due/status and in-process fitting result. |
| CVC-G12 | P1 | i13, i13-2, i4-7 | Add reference-only field “EO load/cycle ID / controlled release record ID” and leave formal release to authorized release documentation, not the in-process checklist. |
| CVC-G13 | P2 | i13-2 | Preserve F-823-i13-2 number and two-page structure in draft. Add controlled approval and actual revision confirmation before effective use. |
| CVC-G14 | P2 | i13-2 | Kit variant/product code; BOM ID/rev; required component/qty; present component/qty; component lot and result; open deviations/approval reference. |

# 4. Test frequency / evidence ownership

| Frequency route | Control owner | Record / requirement |
| --- | --- | --- |
| At designated source time / shift | QC / Production | Appearance, line screening, dimensions and product count; preserve source schedule and approved SOP. |
| Incoming/supplier | Supplier Quality / QC | Purchased accessory certificate, grade, drawing, component lot and approved supplier dossier. |
| Type/qualification | Engineering / QA | Standard method, report and reference connector for qualified product design; no invented routine lot frequency. |
| Periodic / change driven | Engineering / QA | Risk/change assessment and qualifications after tooling/material/sterilization/packaging changes. |
| After EO | QA / Authorized release | Cycle/BI where applicable, final inspection and formal disposition, not an in-process checkbox. |

# 5. Original requirement preservation & approval restrictions

The companion Excel forms retain the source checks and their controlled identities. Any process target without verified approved engineering/supplier specification is shown as a SOURCE COMPANY CRITERION — TECHNICAL CONFIRMATION PENDING. All files are DRAFT candidates; their actual implementation requires controlled revision, training and line qualification where applicable.

# 6. Known outstanding records

OPEN — Supplier product qualification/type-test records

OPEN — Controlled product drawings, DD-02-PS / TF-29-PS and BOM

OPEN — ISO 80369-20 for detailed test methods when applicable

OPEN — EO validation and individual load release evidence

OPEN — Current company controlled form index and exact approved sampling interval

OPEN — Source F-823-i27A where automation is used