# REJECT CONTROL

Controlled procedure draft  /  Saudi Mais Quality System

# 1.  Purpose

To establish one controlled and traceable process for identifying, segregating, documenting, approving and executing the disposition of rejected materials, products and parts from production, Quarantine, OBM and supplier-related sources; to prevent unintended use, transfer or destruction before the applicable decision.

# 2.  Scope

Applicable to Assembly, Injection and Packaging daily production rejects, Quarantine inspection rejects, warehouse-related OBM rejects and supplier-supplied material or parts, from initial detection through authorized disposition and record closure. Separate controls apply if a nonconformity is detected after product delivery or affects released product.

# 3.  Definitions

| Term / record | Definition / meaning |
| --- | --- |
| HOLD QC | Temporary status for material/product pending QC verification or documented disposition; not equivalent to authorization to destroy. |
| Daily reject rate | The area-specific reject percentage determined using the company-approved calculation and reporting method; 2.5% is a company threshold, not an ISO limit. |
| OBM | Out of Box Measurement inspection, following the terminology in the existing REJ procedure. |
| Disposition | Documented decision: Rework, Return to Supplier, Transfer to Warehouse for Destruction, or specifically described Other. |
| F-7-5-3-12-2 | Daily Production & Rejection Record; routine daily production rejection record. |
| F-7-5-7-8 | Rejection Slip for Material / Products / Parts; formal rejection/disposition record used where specified below. |

# 4.  Responsibilities

| Function | Responsibilities |
| --- | --- |
| QC Compliance / Quality Controller | Daily area follow-up, random checks, identification verification, rejection slip preparation, supporting evidence, coordination and closure. |
| Area Supervisor / Production | Segregate rejects, ensure accurate daily production entries and physically verify elevated production rejection; sign when elevated approval is required. |
| Quarantine / Inspection | Provide completed Inspection Report and traceable representative rejected sample; maintain secure status pending disposition. |
| OBM team | Provide inspection/rejection findings and lot / quantity details for warehouse-related rejected items. |
| Purchasing Manager | Evaluate and document whether supplier-related rejected goods can be returned; coordinate authorized supplier return. |
| Quality Manager & Factory Director | Jointly determine and sign formal disposition on F-7-5-7-8 where this form is mandatory; ensure affected materials remain controlled. |
| Warehouse | Receive only authorized transferred rejected goods; record handover and carry out approved destruction within the permitted site process; send evidence to QC. |

# 5.  Procedure

## 5.1  Identification, segregation and HOLD QC

Identify the source (Production, Quarantine, OBM or supplier-related), product code, description, lot/batch, unit, quantity, area and factual reason for rejection.

When a damaged or potentially nonconforming product is identified, immediately segregate it and attach a visible HOLD QC tag. Keep it out of approved inventory and production use until QC documents an applicable routine classification or the required management disposition is signed.

The HOLD QC label shall show at least product/item code, lot, quantity, rejection reason, origin, date, QC contact and the linked record number where applicable.

No person shall release, rework, return to supplier, transfer for destruction or destroy products outside the applicable documented authorization and status-control route.

## 5.2  Routine daily production rejects: Assembly / Injection / Packaging

Daily rejects occur as part of normal operations in Assembly, Injection and Packaging. Normality does not excuse missing quantity/lot records, incorrect classification, unidentified nonconforming product or uncontrolled disposal.

| Daily rate | Required documentation and actions |
| --- | --- |
| Below 2.5% | Use F-7-5-3-12-2 only as the routine daily record. QC Compliance performs random physical/data verification. A separate F-7-5-7-8 is NOT required solely for this routine below-threshold reject. |
| Above 2.5% | Record daily rejects in F-7-5-3-12-2 AND prepare F-7-5-7-8 as the dedicated formal reject report. Obtain Area Supervisor, Quality Manager and Factory Director signatures before irreversible disposition. |
| Exactly 2.5% | The previous REJ Rev. 1 does not define the boundary. Keep affected product on HOLD QC and obtain documented Quality Manager direction pending formal revision of the threshold rule. |

QC random verification includes physical defect, location/tag status, product code, lot number, date, recorded reject reason and accuracy of reported quantities. QC uses the existing approved method for reject-rate calculation; this draft does not create a new denominator.

## 5.3  Rejects originating from Quarantine

Quarantine / Inspection shall provide QC with both the applicable completed Inspection Report and a representative rejected sample traceable to the same item/lot and failure. Missing evidence prevents final disposition submission.

QC verifies the sample, inspection result, product identification, quantity and rejection reason, and prepares F-7-5-7-8 with the Inspection Report and sample identification attached or cross-referenced.

The Quality Manager and Factory Director shall select the disposition and sign the Rejection Slip before the authorized action is implemented.

## 5.4  OBM rejects and warehouse handling

For EVERY OBM reject, QC prepares F-7-5-7-8 regardless of rejection percentage and obtains Quality Manager and Factory Director signed disposition approval.

For OBM rejection below 2.5%, QC retains the original random verification and Focus system traceability entry, in addition to the new mandatory F-7-5-7-8.

For OBM rejection above 2.5%, Area Supervisor sign-off is additionally required, consistent with the original elevated approval route. Use a signed attachment if F-7-5-7-8 has no matching Area Supervisor field.

If the signed decision is destruction, return the identified rejected products to Warehouse. Send an official email attaching the approved F-7-5-7-8 and any supporting report/authorizations. Warehouse performs the authorized site-controlled destruction and supplies completion evidence to QC.

If management explicitly selects another permitted disposition (e.g., approved rework or supplier return), execute that selected path; do not automatically destroy every OBM reject.

## 5.5  Supplier-supplied rejected products

When the rejected item came from an external supplier, QC shall contact the Purchasing Manager before irreversible action. Purchasing shall document supplier-return feasibility and response / reference. Purchasing coordinates the return only after the Quality Manager and Factory Director approve Return to Supplier on the formal rejection record. Commercial acceptance of a return is not quality release authorization.

## 5.6  Formal disposition selection and approval

| Decision branch | Minimum implementation and closure control |
| --- | --- |
| Rework | Use reviewed/approved rework instruction or method; assess potential adverse effects as required by applicable controls; QC reinspects/tests and documents acceptance or further rejection before release. |
| Return to Supplier | Purchasing Manager verifies feasibility; Quality Manager and Factory Director approve; maintain supplier communication, return authorization, shipping and inventory adjustment evidence. |
| Transfer to Warehouse for Destruction | Transfer only the signed-authorized code/lot/quantity; obtain Warehouse receipt, controlled destruction evidence and record reconciliation. |
| Other (specify) | Describe exact action, rationale, responsibilities and closure evidence; seek applicable risk/regulatory review. The “Other” box alone never authorizes unreviewed use-as-is or market release. |

The existing F-7-5-7-8 sample contains Rework, Transfer to Warehouse and Others. Until Document Control approves a form revision, write “Return to Supplier” clearly under Others with the supporting Purchasing record; do not hand-edit the controlled form layout.

## 5.7  Record closure, changes and escalation

Verify that decision, item code, lot, quantity, signatories, handover evidence and actual disposition all match before marking the case closed.

Maintain inspection reports, sample references, Focus entries, daily reports, signed F-7-5-7-8, correspondence, rework verification, supplier return and destruction evidence per the company records-retention process.

Escalate recurrent/significant rejection trends and any potential post-delivery impact through applicable nonconformity, CAPA and regulatory escalation procedures. Do not presume a warehouse OBM case is post-delivery solely because it is in Warehouse.

# 6.  Approval and Evidence Matrix

| Source / condition | Core record | Authorizations | Closure evidence |
| --- | --- | --- | --- |
| Production <2.5% | F-7-5-3-12-2 | QC random verification; no individual F-7-5-7-8 | Verified daily record and controlled reject segregation |
| Production >2.5% | Daily record + F-7-5-7-8 | Area Supervisor + Quality Manager + Factory Director | Signed decision; rework, return or destruction evidence |
| Production =2.5% | Daily record + interim written instruction | Quality Manager documents interim direction | HOLD QC pending authorized decision |
| Quarantine - any rate | Inspection Report + sample + F-7-5-7-8 | Quality Manager + Factory Director | Disposition evidence + sample linkage |
| OBM <2.5% | OBM findings + Focus + F-7-5-7-8 | Quality Manager + Factory Director | Approved return/handover + action evidence |
| OBM >2.5% | OBM findings + F-7-5-7-8 | Area Supervisor + Quality Manager + Factory Director | Authorized handover + action evidence |
| Supplier-origin case | Above as appropriate + Purchasing feedback | Quality Manager + Factory Director on formal disposition | Supplier acceptance / return evidence if applicable |

# 7.  Controlled Forms, Records and Internal Documents

| Identifier / name | Use / status |
| --- | --- |
| F-7-5-3-12-2 - Daily Production & Rejection Record | Routine daily Assembly, Injection and Packaging recording; use controlled current version. |
| F-7-5-7-8 - Rejection Slip for Material / Products / Parts | Formal rejection and disposition; supplied illustrative company form shows Rev. 4 (current revision must be verified). |
| REJ - Reject Handling and Destruction Procedure, Rev. 1 (15.05.2026) | Predecessor source procedure; this draft proposes its replacement after approval. |
| SOP-8-2-6 - Inspection and Testing Procedure | Company example (Rev. 9 pictured), relevant inspection / Quarantine interface; verify current controlled version before issuing cross-reference. |
| Applicable Inspection Report / OBM evidence / Focus entry | Source verification and traceability documents, as applicable. |
| Current Rework, Warehouse Disposal, NCR / CAPA and Supplier Return instructions | Follow if applicable; exact internal document numbers to be verified by Document Control. |

# 8.  Applicable References and Cross-References

| Reference | Relevant control / application |
| --- | --- |
| ISO 13485:2016, 4.2.5 | Control, legibility and retention of rejection, approval and disposition records. |
| ISO 13485:2016, 7.4.1 and 7.4.3 | Purchasing controls and verification of externally provided products; related to supplier communication. |
| ISO 13485:2016, 7.5.8 / 7.5.9 / 7.5.11 | Identification, applicable traceability and preservation during segregation and transfer. |
| ISO 13485:2016, 8.3.1 and 8.3.2 | Control and disposition of nonconforming product detected before delivery. |
| ISO 13485:2016, 8.3.3 | Additional controls for nonconformity identified after delivery, if applicable; warehouse OBM alone is not proof of delivery. |
| ISO 13485:2016, 8.3.4 | Controlled rework with appropriate verification after rework. |
| SFDA MDS-G024 (V1, 25.03.2025) | Saudi regulatory mapping guidance for ISO 13485 requirements; guidance, not a substitute for the standard. |

Clause links are relevance mappings, not claims that ISO requires a 2.5% rejection threshold, a particular company form number, or specific company signatories.

ISO official reference: ISO 13485:2016 publication

SFDA official reference: MDS-G024 guidance

SFDA official PDF: MDS-G024 PDF (2025)

# 9.  Items Requiring Confirmation Before Controlled Release

| Open point | Required management / Document Control action |
| --- | --- |
| Exact 2.5% boundary | Decide whether 2.5% is normal or elevated and approve corresponding revised wording. |
| Roles & signatures | Confirm “Factory Director” replaces earlier “Factory Manager”; define Area Supervisor signature attachment for >2.5% where form lacks field. |
| Form disposition fields | Confirm whether to revise F-7-5-7-8 to add a dedicated Return to Supplier box or retain Other with explicit text. |
| Destruction authorization | Confirm site-approved destruction method, EHS constraints, retention and official warehouse completion evidence. |
| Reject rate method | Verify approved calculation, denominator, time window and owner for each production/OBM measurement. |
| Document Control | Confirm final REJ revision / effective date and latest forms, approval hierarchy and document links. |

# 10.  Modification / Documentation Change History

| Revision | Date | Change / description | Status |
| --- | --- | --- | --- |
| 1 | 15.05.2026 | Original REJ source procedure: production and OBM threshold routes. | Historical |
| 2 (proposed) | 16.10.2026 | Draft alignment: Quarantine, supplier return, QC HOLD, daily form, OBM slip, disposition, approval controls and ISO references. | Not approved |